Technical Overview

The Open Telescope Of Big Bear Solar Observatory (OTBBSO)

# Main specifications of the telescope

- 1.2 m aperture, direct pointing

- Gregorian configuration with cooling systems

▲Open telescope tube, fully retractable dome

▲Equatorial mount

▲Automatic pointing & tracking with Sun guide

▲Adaptive optics

- Focus redirectable into three the plan of instrument

▲3 arcmin field of view

▲42.4m effective focal length. The final ratio f/35.3

▲Wavelength range from 0.3-1.7u

▲High resolution: 0.12arcsec (0.1 arcsec with adaptive optics)

▲Observing moon in night is available

▲Many points monitor temperature for the telescope

▲Primary mirror: Zerodur; parabplic mirror

- Second & third mirror: Silicon carbide (SiC), elipse mirror

▲Adaptive optics systems

NOTE: This design is utilizing the equatorial and declination axis, fork, dome, base and building of  BBSO’s 65 CM telescope

Instruments

★the visible Fabry-perot spectrometer.

★the infrared Fabry-perot spectrometer.

- high resolution  spectrograph

Open telescope

In addition to the atmospheric effects, solar telescopes suffer from heating by sunlight of the optics and the air within the telescope. This causes the image to shiver and become blurred. Modern solar telescope are either vacuum telescope, filled with helium or use careful control of the optic’s temperature to reduce heating of the air in telescope.

Vacuum telescope cannot be built with large apertures because such a design requires a vacuum window of extremely high optical quality, one that is as large as its aperture. It is then the first optical component in the telescope and has to be polished to high optical quality. This is difficult because of the thickness needed to withstand the enormous difference in pressure between the air and the inside of vacuum window. So we think it is difficult to design the vacuum of telescope, If the aperture is more large than 1 meter.

OTBBSO is designed as a completely open telescope. If we want to operate the open telescope very well, key point is keeping the temperature of mirror as same  high as the air , then there is not swift between the surface of mirror and the air .We design a careful cooling system for the purpose, the temperature controlled accuracy±0.1℃.

The layout of open telescope and retractable dome as show below:

## Optics

The telescope uses a two mirrors Gregorian configuration with a f/3.2 primary mirror (M1), the shape of mirror is a parabolic mirror. A tilted cooled head stop mirror (M2) at the primary focus reject the 98% light out to heat stop except a hole 3 arc min in diameter, this is enlarged by an ellipsoidal second mirror (M3) and reflected out of the axis by a flat mirror (M4). The mirror (M5), mirror (M6) and mirror (M7) in fork , there are rotation with azimuth axis, there reflect image in focal plane. In focal plane, we can use another optic systems for the purpose.

The layout of optics as show below

The primary F-RATIO=3.2

The effective focal length = 42.1

The final ratio f/35.5

The focal resolution =1’’/0.206mm

Exit pupil diameter = 225 mm, position is located 872mm behind second mirror

In order to obtain the high resolution of telescope, optics will be arrived diffraction limited, so the primary mirror M1,the second mirror M3, and flat mirror (M4,M5,M6,M7), the polished accuracy RMS is λ/20.

Mechanical structure

We utilize the mount of 65CM telescope of BBSO for the 1.2M telescope. Design a new open tube to fit the mount and design other ports for the telescope. Main problem for this telescope is stiffness, and deformation by temperature change. We consider this problem seriously, use indium in some key parts in the telescope, and adapted equivalent distortion technology, the tube absolute distortion is less than 10 micron. Also we use cooling systems to maintain some parts which face to the sun as same others, when telescope observe.

Control system

We uses PC industrial controlled computer to control the telescope with LINUX operated system.

Uses DC motor to drive the telescope and sun guider for tracing, the absolute encoder be adapted in the systems for high pointing , in this case the telescope also can be used for other propositions.

The dome  moving  with telescope simultaneously. There are four high-precision temperature control systems for four mirrors (M1,M2,M3,M4), temperature control accuracy is ±0.1 ℃. There are three delicate temperature control systems for others components of telescope which needs to reduce effect by temperature, temperature control accuracy is ±0.3 ℃. More sixty the temperature monitor points will be needed in the telescope to estimate the performance by temperature change.

Moving range: AZ axis ±180° (south is origin)

El axis +10°to +80°

Speed range:  AZ axis 1”/sec to 1°/sec

EL axis 1”/sec to 1°/sec

Pointing accuracy:  5”

Tracing accuracy: 1”/5 min (without sun guiding)

1”/10 Min (with sun guiding)

The plan and program of the telescope

The detail process of the telescope as show as below the table, we will spent three years to complete this the project, if we start to work for the telescope in the beginning of 2003, will finishing by end of 2005. We will design, manufacture, assembly and test the telescope for two year and half in China in early days, late we will assembly and test the telescope in U.S.A. for half year.

Total fee: $836,000

This number is rough estimate, we will calculate the fee carefully when we go bake to China, then giving a exactly quoted price.